LED PANEL LIGHT

Item No.:ES-PL-S1230

SPECIFICATION

LED Panel Light

Features:

- Super-thin, super-light and super-brightness.

- New design of optical configuration with high brightness, equal emitting surface, no flare, no flash and high efficiency for energy saving.

- Adapting current circuit design with stable working, reliable effect.

- Using optical panel material to improve the efficiency of light.  High-thermal conductivity aluminum alloy to improve the efficiency of heat conduction.

- Perfect quality SMD, LED as light source with high light efficiency, speed activate, long lifetime.

- No noise and flash when working and frequent activation no affect light source.

- Warm white, natural white and cool white for choosing according to customer’s requirements.

- Beautiful, super- light design, and convenient installation.

- Long-life using can lower maintain cost.

- Different installation for choosing as customer’s requested.

LED panel light specifications:

| LED PANEL SPECIFICATIONS |  |  |
| --- | --- | --- |
| NUMBER | ITEM | DESCRIPTION |
| 1 | Input voltage | AC 100~240V |
| 2 | Panel Input | DC:36-40V，Constant Current:1.05A |
| 3 | Power consumption | 44W |
| 4 | luminous flux | 3592lm |
| 5 | Color | White |
| 6 | Color temperature | 3000-6000K |
| 7 | Life time | >50,000 Hour |
| 8 | Operating temperature | -20ºC to + 50ºC |
| 9 | Storage temperature | -30ºC to + 60ºC |
| 10 | Certificates | CE RoHS FCC |
| 11 | Size | 1195*1195*8.5 mm |
| 12 | Protection Grade | IP20 |

Light Distribution Graph

Panel Size:

Electrical connection diagram:

Installation:

1. Built-in installation

- Remove the ceiling where the LED driver and the panel light will be installed.

- Connect  the  Power  Supply  Cable.

- Carefully insert the LED panel light Along the diagonal direction into the removed ceiling.

- Turn on the AC power to light up the LED panel light.

2. Wall surface mounted

- Install two screws on the wall,center distance as 487.5mm,

- Lock  the  hoist  clamps  into the panel light,

- Put the power supply on the wall or hide it dark channel,

- Put the panel light on the screws and adjust in proper position ,then connect the  power.

3.Pendant installation

- Lock the upper line locks on the roof

- Lock the underneath line locks on the panel light.

- Connect the upper and underneath line locks via steel wire rope, adjust the length, then connect the power.

Package

| 5pcs/Carton |  |
| --- | --- |
| Quantity pcs | each piece in inner box and 5 pcs/carton |
| Weight KG | 21.0 |
| Inner box mm | 1260x360x15 |
| Out carton mm | 1280x380x110 |

Attention

- Please be gentle when take out the product, in order to prevent scratch the surface.

- Please use screwdriver screw by hand,don't use the electrical screwdriver.

- Please stay away from fire or high temperature object when product installation.

- Please turn off  the power before clean the lighting use dry dishcloth to wipe.

- Power supply, controller, tieline etc, to support our company's product, don't match others model.

- Let the children away from the product.

- When install or use the product meet the problem,pls ask the professional to maintain it and don't take apart the product without authorization.